| Schematic Sheet | Description |
| --- | --- |
| Top Sheet |  |
| Power Section |  |
| MCU Core Power | Power Sources - VBAT_SYS – What is the voltage level? - VCCIP0_CORE_DC – What is the voltage level? Components - 6V/3.5A, Fast Response, Adaptive COT Step-Down Converter in Tiny DFN2x2 Datasheet – ETA3451D2I-T https://drive.google.com/file/d/1XjPJf44RNFbel47kCcIjx50fKOjKllRC/view?usp=sharing Description ETA3451 belongs to a new breed of high frequency synchronous Step-Down converters that combines the advantages of voltage mode control and Constant-On-Time control. Its adaptive Constant-On-Time control dynamically changes switch on time to achieve a constant switching frequency. It does not have the minimum on-time constrain normally a fixed-frequency current mode Step-down requires, allowing it to go down to very low duty ratio without affecting loop stability. The voltage mode nature of ETA3451 also provides more superior load transient response and a seamless transition from Pulse Frequency Modulation (PFM) to Pulse Width Modulation (PWM) modes. ETA3451 is capable of supplying output with current up to 3.5A at 1.2V output. All these features make ETA3451 an excellent choice for ARM based CPU power supply. ETA3451 is in a tiny DFN2x2-8Lpackage. Features - Adaptive COT control - Up to 95% Efficiency - Up to 91% Efficiency for low output voltage - Up to 3.5A Max Output current - Feedback voltage 0.45V - Excellent load transient response - DFN2X2-8L Package Typical Application - Circuit of 1.2V Output |
| USB Power | Power Sources - VBAT_USB – What is the voltage level? - LVDS_VDD – What is the voltage level? - VCC3P3_SYS – What is the voltage level? - VBAT_SYS – What is the voltage level? - 2.7A, 1.2MHz, High-Efficiency Step-Up Converter with Input Disconnect Datasheet MP3221GJ-Z https://drive.google.com/file/d/16WZVa2xYAE9JyGfPRwQkRSHuI5i3-Qb_/view?usp=sharing Description The MP3221 is a 1.2MHz frequency, 6-pin TSOT23 current mode step up converter intended for small, low power applications. The device can step up three-cell alkaline, NiCd, and NiMH batteries, or a single-cell Li-on battery up to 6V. It is capable of delivering 1A output current at 5V with a supply voltage of 3V. The MP3221 provides the internal soft start and compensation to minimize the external component count and help producing a compact solution. It integrates a FAULT driver for external PMOS to disconnect the output from input when the part shuts down or in output short circuit condition. This disconnect feature allows the output to be completely discharged, thus allowing the part to draw less than 1μA off current in shutdown mode. The MP3221 is available in a small 6-pin TSOT23 package. Features - Integrated 88mΩ Power MOSFET - 270μA Quiescent Current - 2.5V to 6V Input Voltage - 3V to 6V Output Voltage - 1.2MHz Fixed Switching Frequency - Internal 2.7A Switch Current Limit - Integrated Input Disconnect Driver - Internal Soft Start and Compensation - True Output Disconnect from Input - Under Voltage Lockout - Short-Circuit Protection - Over-Temperature Protection - Available in a 6-Pin TSOT23-6 Package Applications - Single-Cell Lion Battery or Three-Cell Alkaline, NiCd, or NiMH Battery Based Products - Portable Media Players - Wireless Peripherals - Tablets and Smart Phones Diagram - Typical Application - SOT-23 Plastic-Encapsulate MOSFETS Datasheet CJ2305 https://drive.google.com/file/d/1OT3KeYp2cjUi-N5ibpDGFgZ-Yf5CM7c3/view?usp=sharing Feature - TrenchFET Power MOSFET Application - Load Switch for Portable Devices - DC/DC Converter Typical Application |
| RTC Section | Power Sources - VCC 1P8_RTC - What is the voltage level? - VBAT_SYS# - What is the voltage level? - Surface Mount Fast Switching Diode Datasheet 1N4148W-7-F BAT https://drive.google.com/file/d/1pMWrj3LNtGiDIUtWLONlCUWwSaPoR7xA/view?usp=sharing Features - Fast Switching Speed - Low Forward Voltage: Maximum of 0.715V at 1mA - Fast Reverse Recovery: Maximum of 4ns - Low Capacitance: Maximum of 2pF - Surface Mount Package Ideally Suited for Automated Insertion - For General Purpose Switching Applications - Totally Lead-Free & Fully RoHS Compliant (Notes 1 & 2) - Halogen and Antimony Free. “Green” Device (Note 3) - Qualified to AEC-Q101 Standards for High Reliability - PPAP Capable (Note 4) - Ultra-Low Current Consumption and Low Dropout Cmos Voltage Regulator Datasheet S-1206B18-M3T1U https://drive.google.com/file/d/11JNy0JxpWSveVUC_rDiVuLjXPk1y6toH/view?usp=sharing Description The S-1206 Series is a positive voltage regulator with ultra-low current consumption, low dropout voltage, high-accuracy output voltage, and 250 mA output current developed based on CMOS technology. I/O capacitors are as small as 0.1 F. S-1206 Series operates at ultra-low current consumption of 1.0 A (typ.). The built-in low-on-resistance transistor realizes low dropout voltage and a large output current. A built-in overcurrent protection circuit prevents the load current from exceeding the current capacity of the output transistor. Three packages, SOT-23-3, SOT-89-3, and SNT-6A(H) are available. Compared with voltage regulators using a conventional CMOS technology, more types of capacitors, including small I/O capacitors, can be used with the S-1206 Series. The S-1206 Series features ultra-low current consumption and comes in a small package, making them most suitable for portable equipment. Features - Output voltage: 1.2 V to 5.2 V, selectable in 0.05 V step - Input voltage: 1.7 V to 6.5 V - Output voltage accuracy: 1.0% (1.2 V to 1.45 V output product: 15 mV) - Dropout voltage: 150 mV typ. (3.0 V output product, IOUT = 100 mA) - Current consumption: During operation: 1.0 A typ., 1.5 A max. - Output current: Possible to output 250 mA (3.0 V output product, VIN VOUT(S) 1.0 V) *1 - Input and output capacitors: A ceramic capacitor of 0.1 F or more can be used. - Built-in overcurrent protection circuit: Limits overcurrent of output transistor. - Operation temperature range: Ta = 40°C to 85°C - Lead-free, Sn 100%, halogen-free*2 Applications - Constant-voltage power supply for battery-powered device - Constant-voltage power supply for cellular phone - Constant-voltage power supply for portable equipment Typical Application |
| Audio Section |  |
| MIC | Datasheet POM-2738P-R |
| Audio | Power Sources LVDS_VDD VCC3P3_SYS Datasheet ALC5621 Datasheet S2B-XH-A(LF)(SN) JACK |
| Headphone | Power Sources VCC3P3_SYS Datasheet SJ-43504-SMT-TR |
| 3G Section |  |
| 3G Modem | Power Sources VCC3P3_SYS VBAT Datasheet SIM7100AS2-10671 Datasheet U.FL-R-SMT-1(80) |
| USIM | Power Sources VBAT VBAT_USB Datasheet 693043020611 Datasheet ESDA6V1-5SC6 |
| Level Shifter | Power Sources VCC3P3_SYS VCC IP8_LDO Datasheet S-1206B18-M3T1U |
| Controller |  |
| Main Controller | Datasheet S5P4418 |
| Camera Section | Datasheet 0524352471 |
| RS 232 Transceiver | Datasheet SP3222EEY-L/TR Datasheet A-DF 09 A/KG-T4S |
| LCD Section | Datasheet 4-1734592-2 |
| Connector Section | Power Sources VBAT |
| Memory Section | Power Sources VCC3P3_SYS Datasheet 114-00841-68 |
| USB Section |  |
| USB OTG | Power Sources DC5V_OTG VBAT_USB Datasheet SY6280AAC Datasheet 10118194-0001LF |
| USB Host | Datasheet GL850G Datasheet 87583-2010BLF |
| Input Section |  |
| Key Buttons |  |
| Sensor Section | Power Sources VCC 1P8_LDO VCC3P3_SYS Datasheet ICM-42688-P Datasheet VEML6030-GS15 |